Program:      gdsserver

Purpose:      Handles GDS requests from GIPSY tasks.

Category:     SYSTEM

File:         gdsserver.src

Author:       J.P. Terlouw

Use:          $gip_exe/gdsserver

Description:  The server is normally started by the first task which needs it.

This is done transparantly when any GDS routine is called.

After startup it can also process requests from other tasks.

For every user who runs one or more GIPSY sessions there is

a server for every directory in which sets are being accessed.

Tasks can communicate with the server in one of two ways:

via a Unix socket or via an Internet socket. Using the Unix

socket is the most efficient. This method is chosen when the

server is started and will be used by all subsequent clients

which run on the same host. Clients on other hosts will use

the Internet socket.

When the server does not have any clients anymore, it will

exit after 5 minutes, except if the server has only had remote

clients during the last 5 minutes. In this case it will exit

immediately so that clients have the opportunity to start

a local server with which communication is more efficient.

To connect to the server, a task uses the following files in the

user's home directory, which are created by the server:

.gds_sockets<directory_id>

.gds_read

.gds_write

.gds_sockets* contain information about the Internet address of

the host on which the server runs, the Internet port number and

the path of the Unix socket. This file is only present when the

server is active.

.gds_read contains a 'password' which the client software needs

to read from a GDS set using the server. The protection of this

file determines who can access the server to read from any sets

it serves. The default protection of this file is 644, but this

can be changed by the user with the chmod command.

.gds_write contains a 'password' which the client software needs

to write to a GDS set using the server. The default protection

of this file is 600, but this can also be changed.

In every directory which is being or has been served, there is

a file with the name .gds_ident .

This version only serves descriptor files. Image I/O is done

by a routine in the client part of GDS.

Updates:      Apr  9, 1994: JPT original document created.

Mar 17, 1995: JPT fixed byte-swap bug in DescrFind (descr.c);

added server id command line argument.

Mar 28, 1995: JPT do not change existing .gds_read and .gds_write

May  3, 1995: JPT improved consistency checks (descr.c).

Aug 24, 1995: JPT inter-architecture bug fixed (clientmgr.c).

Mar 26, 1996: JPT null pointer bug fixed (linkedlist.c).

Sep  2, 1996: JPT Small code change by KGB.

Jun 26, 1998: JPT Fixed bug in low-high-endian inter-operability.

Sep 19, 2000: JPT Exit on sockets file write error.

Feb  8, 2002: JPT Fixed byte-swap bug in DescrDelall (descr.c).

May  4, 2007: JPT Included conditional code for Apple Mac.

Feb 19, 2009: JPT Implemented temporary (FITS pseudo-) sets.